20 OBJECTIVE TYPES QUESTION- Entry Level

1. During confined space entry, the oxygen level is found as 20.0%. What is the most appropriate action?
A) Allow entry because oxygen is above 19.5%
B) Start work with continuous gas monitoring
C) Improve ventilation and verify atmosphere before entry
D) Allow entry with SCBA only without further action
Correct Answer: C

2. A hot work permit is approved, but during welding activity LEL reaches 10%. What should the Safety Engineer do first?
A) Increase gas monitoring frequency and continue work
B) Stop work, remove ignition source, and reassess atmosphere
C) Allow work because LEL is below 20%
D) Continue work with additional fire extinguisher
Correct Answer: B

3. The main reason for conducting a JSA before starting a task is to:
A) Identify hazards and define controls before exposure occurs
B) Record workers involved in the activity
C) Confirm availability of PPE and tools
D) Meet client documentation requirements
Correct Answer: A

4. During lifting operation, the load starts swinging unexpectedly. The immediate action should be:
A) Increase crane speed to stabilize the load
B) Use tag lines and continue lifting slowly
C) Stop the operation and control the load safely
D) Reduce load weight after completing the lift
Correct Answer: C

5. A confined space has been tested and declared safe. During entry, ventilation stops. What is the correct response?
A) Continue work because initial testing was acceptable
B) Exit the space and re-establish safe conditions
C) Continue using escape respirator
D) Increase work speed and complete the task
Correct Answer: B

6. Which statement best describes a Permit to Work system?
A) A document allowing workers to perform any activity safely
B) A formal communication system controlling hazardous work
C) A replacement for risk assessment and toolbox talk
D) A document confirming workers are trained
Correct Answer: B

7. A scaffold has a green tag but workers notice missing mid-rails. What should be done?
A) Use scaffold because tag is approved
B) Report defect and stop use until corrected
C) Use scaffold with additional PPE
D) Allow use for short duration only
Correct Answer: B

8. Excavation work is ongoing and cracks appear near the edge. The primary concern is:
A) Soil erosion only
B) Possible excavation collapse due to instability
C) Increase in water accumulation
D) Reduced excavation depth
Correct Answer: B

9. In LOTO procedure, verification of zero energy is required to:
A) Confirm equipment is switched off
B) Confirm stored energy has been isolated
C) Confirm the lock color is correct
D) Confirm maintenance team is available
Correct Answer: B

10. The most reliable method to prevent falls from height is:
A) Safety harness with proper anchorage
B) Guardrails and fall prevention systems
C) Safety helmet and warning signs
D) Worker awareness training
Correct Answer: B

11. A gas detector shows 5% LEL during routine inspection. The best action is:
A) Continue work with monitoring
B) Stop activity and investigate gas source
C) Increase ventilation only
D) Issue new work permit
Correct Answer: B

12. The purpose of a lifting plan is mainly to:
A) Define lifting method, hazards, and controls
B) Record crane inspection details only
C) Replace operator competency verification
D) Reduce lifting duration
Correct Answer: A

13. Which control is considered strongest for reducing chemical exposure?
A) Chemical-resistant gloves
B) Local exhaust ventilation
C) Replacing hazardous chemical with safer alternative
D) Safety training
Correct Answer: C

14. A worker enters an excavation without checking underground services. The main failure is:
A) PPE selection failure
B) Planning and hazard identification failure
C) Communication failure only
D) Housekeeping failure
Correct Answer: B

15. The purpose of emergency drills is to:
A) Test response capability and identify weaknesses
B) Fulfill client requirements only
C) Reduce emergency equipment cost
D) Train only emergency team members
Correct Answer: A

16. During welding, a fire watch person's primary responsibility is:
A) Assist welder with equipment
B) Monitor fire hazards during and after hot work
C) Issue hot work permit
D) Control welding parameters
Correct Answer: B

17. A near miss investigation is important because it:
A) Identifies failures before a serious incident occurs
B) Is required only when property damage occurs
C) Replaces accident investigation
D) Determines employee punishment
Correct Answer: A

18. The best indication that a safety program is effective is:
A) Number of safety meetings conducted
B) Reduction in incidents through improved controls
C) Amount of PPE issued
D) Number of safety observations recorded
Correct Answer: B

19. During nitrogen purging, the biggest hidden hazard is:
A) High pressure release
B) Oxygen deficiency causing unconsciousness
C) Low temperature exposure
D) Increased fire risk
Correct Answer: B

20. A worker repeatedly violates safety procedures despite training. The next step should be:
A) Provide another toolbox talk only
B) Investigate behavioural and system causes
C) Increase PPE requirements
D) Ignore if no incident happened
Correct Answer: B

Mid & tough Level Question

1. During a shutdown, production management requests that hot work continue despite one gas detector being out of service. Three other calibrated detectors are operating normally. What is the BEST course of action?

A. Allow the work because sufficient monitoring is still available.

B. Suspend the work until the risk assessment confirms monitoring remains adequate and permit conditions are reviewed.

C. Continue the work while assigning an additional fire watch.

D. Continue the work because the remaining detectors provide adequate redundancy.

✅ Correct Answer: B

Reason: Any change to the assumptions under which the permit was issued requires reassessment through the Management of Change (MOC) and permit review.

2. During an audit, you find that workers consistently follow procedures, yet near misses continue to occur. What is the MOST appropriate management response?

A. Increase disciplinary action for unsafe acts.

B. Increase the frequency of toolbox talks.

C. Evaluate whether existing procedures, controls, and work planning effectively manage the risks.

D. Retrain all workers on the procedures.

✅ Correct Answer: C

3. An investigation concludes that the immediate cause of an incident was worker error. As the HSE Manager, what should be your PRIMARY focus?

A. Reinforce worker accountability.

B. Review competence, supervision, work systems, and organizational factors that contributed to the error.

C. Issue disciplinary action to prevent recurrence.

D. Increase inspections of similar work.

✅ Correct Answer: B

4. Two contractors are performing simultaneous operations (SIMOPS). Both have valid permits, but their activities may interact. What is the BEST management action?

A. Allow both activities because permits have been approved.

B. Review the SIMOPS interface, reassess combined risks, and coordinate controls before work proceeds.

C. Assign additional safety officers.

D. Increase gas testing frequency.

✅ Correct Answer: B

5. During a site visit, production management asks you to approve bypassing an interlock to avoid project delays. What should you do?

A. Approve the bypass if operations agree.

B. Approve the bypass after informing senior management.

C. Permit the bypass only after a formal Management of Change (MOC), engineering review, risk assessment, and authorization.

D. Reject every request to bypass safety systems.

✅ Correct Answer: C

6. A contractor has an excellent injury record but several serious permit violations. What should concern management the MOST?

A. The low injury rate.

B. The possibility that lagging indicators are masking major accident risks.

C. The contractor's experience.

D. The quality of PPE being used.

✅ Correct Answer: B

7. Which KPI provides the BEST indication of future major accident prevention?

A. Total Recordable Injury Rate (TRIR)

B. Lost Time Injury Frequency (LTIF)

C. Percentage of critical control verification completed effectively

D. Number of toolbox talks conducted

✅ Correct Answer: C

8. During an internal audit, documentation is excellent, but field observations show poor implementation. What should be concluded?

A. The HSE Management System is effective.

B. Compliance exists because documentation is complete.

C. System implementation and verification are ineffective despite documented compliance.

D. Additional documentation should be developed.

✅ Correct Answer: C

9. Which management action demonstrates the strongest safety leadership?

A. Personally conducting toolbox talks.

B. Enforcing disciplinary procedures consistently.

C. Ensuring resources, visible leadership, and accountability for critical risk controls.

D. Increasing the number of inspections.

✅ Correct Answer: C

10. A supervisor pressures workers to complete work before permit extensions expire. What is the PRIMARY management concern?

A. Permit duration.

B. Worker competence.

C. Organizational culture prioritizing production over safety.

D. Poor housekeeping.

✅ Correct Answer: C

11. During a major lifting operation, all procedural requirements are satisfied, but weather conditions are becoming increasingly unstable. What is the BEST decision?

A. Continue because the lift has already started.

B. Continue while monitoring wind speed.

C. Suspend the lift and reassess conditions before continuing.

D. Allow completion if the crane operator agrees.

✅ Correct Answer: C

12. Which statement BEST reflects the purpose of Management of Change (MOC)?

A. Managing project schedules.

B. Controlling any change that could introduce new hazards or increase existing risks.

C. Updating procedures annually.

D. Managing contractor performance.

✅ Correct Answer: B

13. A site consistently achieves excellent audit scores but experiences repeated process safety events. What is the MOST likely explanation?

A. Audits are too difficult.

B. The audit process emphasizes documentation over verification of critical controls.

C. Workers require more PPE.

D. The reporting system is ineffective.

✅ Correct Answer: B

14. Which action demonstrates the highest level of risk management maturity?

A. Investigating every incident.

B. Conducting monthly inspections.

C. Verifying that critical controls remain effective throughout the work.

D. Increasing disciplinary action.

✅ Correct Answer: C

15. During emergency response exercises, workers perform well because they already know the scenario. What should management do?

A. Continue using the same drill.

B. Increase the number of drills.

C. Introduce realistic, unannounced, and varied scenarios to evaluate actual readiness.

D. Reduce drill frequency.

✅ Correct Answer: C

16. Which management decision BEST demonstrates the application of the ALARP principle?

A. Accepting any risk below the legal limit.

B. Reducing risk until further reduction is grossly disproportionate to the benefit achieved.

C. Eliminating every identified hazard regardless of practicality.

D. Relying primarily on PPE to reduce risk.

✅ Correct Answer: B

17. A contractor repeatedly meets productivity targets but frequently requests permit deviations. What should management conclude?

A. The contractor is highly efficient.

B. The permit system is too restrictive.

C. There may be underlying planning and risk management deficiencies.

D. Additional supervisors are required.

✅ Correct Answer: C

18. Which indicator is MOST useful for identifying weaknesses before a major accident occurs?

A. Number of first aid cases.

B. Number of lost-time injuries.

C. Critical control failures and barrier performance monitoring.

D. Overtime hours worked.

✅ Correct Answer: C

19. Following an incident, management immediately introduces new procedures without identifying why existing procedures failed. What is the MOST significant weakness?

A. Poor communication.

B. Failure to identify systemic root causes before implementing corrective actions.

C. Lack of worker consultation.

D. Delayed reporting.

✅ Correct Answer: B

20. Which statement BEST reflects the role of an HSE Manager in a high-reliability organization?

A. Ensure workers follow procedures.

B. Conduct audits and inspections.

C. Build systems that anticipate failures, strengthen critical controls, and continuously improve organizational resilience.

D. Ensure legal compliance only.

✅ Correct Answer: C

NEBOSH IGC RELATED QUESTION

1. A supervisor notices an employee repeatedly bypassing a machine guard to increase production. What should be the FIRST management action?

A. Stop the work and discipline the employee immediately.

B. Stop the work and investigate why the guard is being bypassed before deciding corrective actions.

C. Replace the machine guard with a different design.

D. Retrain all employees on machine guarding.

✅ Correct Answer: B

2. Which option BEST demonstrates the application of the hierarchy of control?

A. Providing workers with cut-resistant gloves.

B. Installing machine guarding to prevent contact with moving parts.

C. Displaying warning signs around machinery.

D. Conducting monthly safety training.

✅ Correct Answer: B

3. During a risk assessment, a hazard cannot reasonably be eliminated. What should be considered NEXT?

A. Administrative controls.

B. Personal protective equipment.

C. Engineering controls.

D. Increased supervision.

✅ Correct Answer: C

4. A worker reports a near miss involving a falling object. What is the PRIMARY purpose of investigating the near miss?

A. Identify the employee responsible.

B. Prevent a similar event from resulting in injury.

C. Complete legal documentation.

D. Demonstrate management commitment.

✅ Correct Answer: B

5. Which factor MOST influences whether employees follow safe working procedures?

A. Availability of written procedures.

B. Visible commitment from management.

C. Frequency of inspections.

D. Number of toolbox talks.

✅ Correct Answer: B

6. Which statement BEST describes a "safe system of work"?

A. A written procedure issued before work starts.

B. A formal method of controlling hazards by integrating procedures, competence, supervision, and emergency arrangements.

C. A permit to work issued by a supervisor.

D. A risk assessment completed before work begins.

✅ Correct Answer: B

7. Which of the following provides the STRONGEST evidence that a safety culture is improving?

A. More PPE is being issued.

B. Employees actively report hazards and near misses without fear of blame.

C. More warning signs are installed.

D. Audit scores continue to improve.

✅ Correct Answer: B

8. A company has experienced several similar manual handling injuries. What should management do FIRST?

A. Purchase lifting belts.

B. Review the manual handling risk assessment and redesign the task where possible.

C. Provide refresher training.

D. Introduce disciplinary action.

✅ Correct Answer: B

9. Why should employees be consulted during risk assessment?

A. They approve legal compliance.

B. They have practical knowledge of the work and associated hazards.

C. They can replace the safety officer.

D. They are responsible for preparing procedures.

✅ Correct Answer: B

10. Which factor would MOST likely reduce human error?

A. Increasing disciplinary action.

B. Improving workplace design to reduce reliance on memory.

C. Increasing inspection frequency.

D. Providing more PPE.

✅ Correct Answer: B

11. A contractor has submitted all required safety documents but repeatedly fails to follow site rules. Which is the MOST important concern?

A. Documentation quality.

B. Implementation of the safety management system.

C. Contractor experience.

D. Number of workers employed.

✅ Correct Answer: B

12. Which of the following is the BEST example of a leading performance indicator?

A. Lost Time Injury Frequency Rate.

B. Number of reportable accidents.

C. Percentage of planned workplace inspections completed.

D. Number of compensation claims.

✅ Correct Answer: C

13. Which statement BEST explains the purpose of active monitoring?

A. Measuring accidents after they occur.

B. Identifying whether risk control measures are functioning before incidents happen.

C. Investigating incidents.

D. Recording employee injuries.

✅ Correct Answer: B

14. During an emergency drill, workers follow procedures perfectly because they knew the scenario in advance. What is the MAIN weakness?

A. The emergency plan is too detailed.

B. The drill may not accurately assess emergency preparedness.

C. Workers require additional PPE.

D. The drill should be cancelled.

✅ Correct Answer: B

15. Which management action is MOST likely to improve contractor safety performance?

A. Conduct more audits after incidents.

B. Select contractors based on competence, monitor performance, and review compliance throughout the contract.

C. Issue more warning notices.

D. Increase financial penalties.

✅ Correct Answer: B

16. An investigation identifies "worker failed to follow procedure" as the root cause. Why is this conclusion usually insufficient?

A. Workers are always responsible.

B. It may overlook organizational and management failures that influenced the behaviour.

C. Procedures are rarely effective.

D. Human error cannot be investigated.

✅ Correct Answer: B

17. Which statement BEST reflects the purpose of safety inspections?

A. Identify unsafe conditions and verify control measures remain effective.

B. Replace formal risk assessments.

C. Monitor employee attendance.

D. Prepare for external audits.

✅ Correct Answer: A

18. Which option BEST demonstrates continual improvement within a health and safety management system?

A. Updating policies annually regardless of performance.

B. Reviewing performance data, identifying weaknesses, and implementing effective corrective actions.

C. Increasing inspection frequency every year.

D. Purchasing additional PPE.

✅ Correct Answer: B

19. Following a serious accident, management introduces several new procedures without reviewing existing controls. What is the MAIN weakness?

A. Insufficient training.

B. Failure to identify why existing controls were ineffective.

C. Poor communication.

D. Delayed reporting.

✅ Correct Answer: B

20. According to the Plan–Do–Check–Act (PDCA) model, which activity belongs to the "Check" stage?

A. Setting health and safety objectives.

B. Implementing control measures.

C. Monitoring performance and auditing the management system.

D. Revising the health and safety policy.

✅ Correct Answer: C

MID LEVEL NEBOSH QUIZ

1. Following a fatal incident, the investigation identifies that the written procedure was adequate, but employees consistently did not follow it. What should management investigate FIRST?

A. Whether employees received refresher training.

B. Why the procedure was not practical or consistently followed in the workplace.

C. Whether disciplinary action should be taken.

D. Whether the procedure should be rewritten.

✅ Correct Answer: B

2. During a risk assessment, an existing engineering control reduces the likelihood of injury but does not eliminate the hazard. The next step should be to:

A. Introduce PPE immediately.

B. Determine whether additional reasonably practicable controls are required.

C. Accept the residual risk because engineering controls are already in place.

D. Replace engineering controls with administrative controls.

✅ Correct Answer: B

3. Which action BEST demonstrates that senior management is fulfilling its leadership responsibilities?

A. Personally delivering every toolbox talk.

B. Providing adequate resources, setting clear expectations, and regularly reviewing H&S performance.

C. Conducting daily workplace inspections.

D. Approving all permits to work.

✅ Correct Answer: B

4. An employee follows every written procedure but is still injured. What is the MOST likely conclusion?

A. The employee made an unsafe act.

B. Existing risk controls may not have been sufficient or effective.

C. Additional PPE is required.

D. More disciplinary action is necessary.

✅ Correct Answer: B

5. During an audit, documentation is complete and compliant, but field observations reveal workers regularly bypassing controls. What is the BEST conclusion?

A. The audit confirms an effective management system.

B. The organization demonstrates compliance but lacks effective implementation.

C. Workers require additional training only.

D. Documentation should be simplified.

✅ Correct Answer: B

6. Which statement BEST reflects the concept of "reasonably practicable"?

A. Eliminating every hazard regardless of cost.

B. Balancing the level of risk against the time, effort, and cost required to reduce it.

C. Applying PPE whenever elimination is impossible.

D. Complying only with minimum legal requirements.

✅ Correct Answer: B

7. A company reports zero lost-time injuries for three years but experiences several dangerous occurrences. What should concern management MOST?

A. Injury reporting accuracy.

B. The effectiveness of controls for major hazards despite good injury statistics.

C. Employee morale.

D. The company's audit schedule.

✅ Correct Answer: B

8. Which management action would MOST strengthen a positive safety culture?

A. Increasing disciplinary action for every unsafe act.

B. Encouraging open reporting of hazards without fear of blame while ensuring accountability.

C. Increasing the number of safety signs.

D. Making PPE mandatory in all areas.

✅ Correct Answer: B

9. An investigation concludes that "lack of training" caused an accident. Why might this conclusion be inadequate?

A. Training is rarely effective.

B. It may overlook deficiencies in supervision, planning, equipment, or organizational controls.

C. Workers should already know safe practices.

D. Training records may be inaccurate.

✅ Correct Answer: B

10. Which is the BEST example of an active monitoring activity?

A. Investigating a lost-time injury.

B. Reviewing inspection findings to verify control measures remain effective.

C. Calculating annual accident rates.

D. Recording insurance claims.

✅ Correct Answer: B

11. A supervisor regularly ignores permit conditions because no incidents have occurred. This behaviour primarily indicates:

A. Competent supervision.

B. Normalization of deviance.

C. Effective risk management.

D. Improved productivity.

✅ Correct Answer: B

12. Which factor MOST influences human reliability during complex tasks?

A. Length of service alone.

B. Task design, workload, competence, supervision, and working environment.

C. PPE availability.

D. Number of toolbox talks attended.

✅ Correct Answer: B

13. Which action BEST demonstrates the "Check" stage of the PDCA cycle?

A. Developing objectives.

B. Measuring performance against planned arrangements and verifying effectiveness.

C. Introducing new procedures.

D. Revising organizational policy.

✅ Correct Answer: B

14. Why should organizations investigate near misses with the same systematic approach as accidents?

A. They are legal requirements in every country.

B. They provide opportunities to identify weaknesses before harm occurs.

C. Insurance companies require it.

D. They reduce paperwork later.

✅ Correct Answer: B

15. During contractor selection, which factor provides the STRONGEST evidence of future safety performance?

A. Lowest contract price.

B. Demonstrated competence, effective management systems, and previous performance.

C. Number of employees.

D. Length of time in business.

✅ Correct Answer: B

16. Which management decision BEST demonstrates continual improvement?

A. Increasing inspection frequency after every incident.

B. Reviewing performance data, identifying trends, and improving critical controls based on evidence.

C. Conducting annual policy reviews only.

D. Increasing disciplinary procedures.

✅ Correct Answer: B

17. A worker deliberately ignores a safety procedure because production targets are prioritized by supervisors. The PRIMARY underlying cause is:

A. Worker negligence.

B. Organizational culture and management priorities.

C. Inadequate PPE.

D. Lack of competence.

✅ Correct Answer: B

18. Why are leading indicators generally considered more valuable than lagging indicators?

A. They eliminate accidents completely.

B. They provide information about the effectiveness of controls before failures occur.

C. They replace accident investigations.

D. They require less data collection.

✅ Correct Answer: B

19. A company introduces a new chemical into production without reviewing existing risk assessments. Which management process has MOST likely failed?

A. Emergency planning.

B. Management of Change.

C. Incident reporting.

D. Active monitoring.

✅ Correct Answer: B

20. Which statement BEST describes the role of a health and safety management system?

A. To ensure compliance with legislation only.

B. To systematically manage risks through planning, implementation, monitoring, review, and continual improvement.

C. To reduce insurance costs.

D. To provide written procedures for employees.

✅ Correct Answer: B